# Formulation Trial 02 — Bisoprolol 5 mg Tablet (Direct Compression)

## Trial Overview

| Attribute | Value |
| --- | --- |
| Trial ID | BIS-5-DC-002 |
| Formulation code | F-02 |
| Active ingredient | Bisoprolol fumarate 5 mg |
| Granulation method | Direct compression |
| Batch size | 10,000 tablets |
| Date | 2025-06-22 |
| Operator | Nguyễn Văn A |

## Formulation Composition

| # | Ingredient | mg/tablet | % w/w | Function |
| --- | --- | --- | --- | --- |
| 1 | Bisoprolol fumarate | 5.00 | 5.00 | Active |
| 2 | Microcrystalline cellulose (DG grade) | 60.00 | 60.00 | Filler/binder (direct compressible) |
| 3 | Lactose monohydrate (DCL-11) | 20.00 | 20.00 | Filler (direct compressible) |
| 4 | Croscarmellose sodium | 5.00 | 5.00 | Disintegrant (superdisintegrant) |
| 5 | Colloidal silicon dioxide | 1.50 | 1.50 | Glidant |
| 6 | Magnesium stearate | 1.50 | 1.50 | Lubricant |
| Total |  | 93.00 | 100.00 |  |

## Coating

| Attribute | Value |
| --- | --- |
| Coating material | Opadry II White |
| Coating weight gain | 3.0% w/w |
| Coating pan | 12-inch, inlet temp 60°C |

## Results

| Test | Specification | Result | Pass/Fail |
| --- | --- | --- | --- |
| Appearance | White, round, biconvex film-coated tablet | Conforms | Pass |
| Average weight (n=20) | 93.0 mg ± 5% | 93.4 mg (RSD 1.8%) | Pass |
| Hardness (n=10) | 40–100 N | 52 N (range 42–65) | Pass |
| Friability (n=20) | ≤ 1.0% | 0.68% | Pass |
| Disintegration | ≤ 15 min | 6 min 45 sec | Pass |
| Assay (HPLC) | 95.0–105.0% | 98.8% | Pass |
| Content uniformity (n=10) | AV ≤ 15.0 | AV = 11.2 | Pass |
| Dissolution (Q=80%, 30 min, n=6) | Each unit ≥ 80% | 85%, 88%, 82%, 86%, 84%, 89% | Pass |
| Dissolution mean | — | 85.7% | — |
| Related substances | Individual ≤ 0.5%; Total ≤ 1.0% | Max individual 0.15%; Total 0.42% | Pass |
| Loss on drying | ≤ 2.0% | 1.1% | Pass |

## Observations

- Direct compression showed poorer flow than wet granulation (Carr index 24%).

- Dissolution is slower than F-01 but still meets specification.

- Hardness range is wider — compression force control is more critical.

- Higher disintegrant level (5%) compensates for lack of granule bonding.

- Content uniformity AV is higher (11.2 vs 6.8), indicating mixing is less homogeneous.

- Simplified process (fewer steps) reduces manufacturing time by ~40%.