# Formulation Trial 03 — Bisoprolol 10 mg Tablet (Wet Granulation)

## Trial Overview

| Attribute | Value |
| --- | --- |
| Trial ID | BIS-10-WG-003 |
| Formulation code | F-03 |
| Active ingredient | Bisoprolol fumarate 10 mg |
| Granulation method | Wet granulation |
| Batch size | 10,000 tablets |
| Date | 2025-07-01 |
| Operator | Trần Thị B |

## Formulation Composition

| # | Ingredient | mg/tablet | % w/w | Function |
| --- | --- | --- | --- | --- |
| 1 | Bisoprolol fumarate | 10.00 | 9.09 | Active |
| 2 | Microcrystalline cellulose (PH-102) | 55.00 | 50.00 | Filler/binder |
| 3 | Lactose monohydrate (200 mesh) | 25.00 | 22.73 | Filler |
| 4 | Povidone K30 (binder solution 5% w/v) | 8.00 | 7.27 | Binder |
| 5 | Croscarmellose sodium | 4.00 | 3.64 | Disintegrant |
| 6 | Colloidal silicon dioxide | 1.00 | 0.91 | Glidant |
| 7 | Magnesium stearate | 1.00 | 0.91 | Lubricant |
| 8 | Iron oxide yellow | 0.50 | 0.45 | Colorant |
| Total |  | 104.50 | 100.00 |  |

## Coating

| Attribute | Value |
| --- | --- |
| Coating material | Opadry II Yellow (hypromellose, titanium dioxide, iron oxide, macrogol) |
| Coating weight gain | 3.5% w/w |
| Coating pan | 12-inch, inlet temp 60°C |

## Results

| Test | Specification | Result | Pass/Fail |
| --- | --- | --- | --- |
| Appearance | Yellow, round, biconvex film-coated tablet | Conforms | Pass |
| Average weight (n=20) | 104.5 mg ± 5% | 104.2 mg (RSD 1.0%) | Pass |
| Hardness (n=10) | 40–100 N | 72 N (range 65–82) | Pass |
| Friability (n=20) | ≤ 1.0% | 0.25% | Pass |
| Disintegration | ≤ 15 min | 3 min 50 sec | Pass |
| Assay (HPLC) | 95.0–105.0% | 100.1% | Pass |
| Content uniformity (n=10) | AV ≤ 15.0 | AV = 5.4 | Pass |
| Dissolution (Q=80%, 30 min, n=6) | Each unit ≥ 80% | 96%, 98%, 95%, 97%, 96%, 99% | Pass |
| Dissolution mean | — | 96.8% | — |
| Related substances | Individual ≤ 0.5%; Total ≤ 1.0% | Max individual 0.08%; Total 0.28% | Pass |
| Loss on drying | ≤ 2.0% | 0.6% | Pass |

## Observations

- Higher dose (10 mg) gives better content uniformity (AV 5.4) due to lower relative potency variation.

- Dissolution is fastest among all three formulations — higher drug load creates more porous granules.

- Hardness is highest (72 N) — good mechanical stability.

- Binder level increased (8 mg vs 5 mg) to accommodate higher drug loading.

- Yellow coating distinguishes 10 mg from 5 mg (white) for patient safety.