3.2.P 		DRUG PRODUCT

3.2.P.2 	PHARMACEUTICAL DEVELOPMENT

The following pharmaceutical development report summarizes the development of Generic <<API 5 mg film coated tablets>>, a generic version of the reference medicinal products (RMP), <<Concor®>>. The RMP is <<an immediate release (IR) tablet>> indicated for <<treatment of stable chronic heart failure with reduced systolic left ventricular function>>. We used Quality by Design (QbD) to develop generic <<API IR tablets>> that are therapeutically equivalent to the RMP.

Initially, the quality target product profile (QTPP) was defined based on the properties of the drug substance, characterization of the RMP product. Our investigation during pharmaceutical development focused on those CQAs that could be impacted by a realistic change to the drug product formulation or manufacturing process. For generic <<API tablets>>, these CQAs included <<assay, content uniformity, dissolution and degradation products>>.

TABLE OF CONTENTS:

List of abbreviations:	4

3.2.P.2.1   	COMPONENTS OF THE DRUG PRODUCT	6

3.2.P.2.1.1   DRUG SUBSTANCE	6

3.2.P.2.1.1.1	Physicochemical properties	6

3.2.P.2.1.1.2	Biological properties.	8

3.2.P.2.1.2   EXCIPIENTS	8

3.2.P.2.1.2.1     Physicochemical properties	8

3.2.P.2.1.2.2    Excipient compatibility	9

3.2.P.2.2         DRUG PRODUCT	11

3.2.P.2.2.1   FORMULATION DEVELOPMENT	11

3.2.P.2.2.1.1  Characterization of Reference Product	11

3.2.P.2.2.1.2  Quality Target Product Profile, CQAs for the Generic Product.	12

3.2.P.2.2.1.3  Formulation Development Study	18

3.2.P.2.2.2   OVERAGES	28

3.2.P.2.2.3   PHYSICOCHEMICAL AND BIOLOGICAL PROPERTIES	29

3.2.P.2.2.3.1. Development of a dissolution method	29

3.2.P.2.2.3.2. Impurities profile of tablets	36

3.2.P.2.3         MANUFACTURING PROCESS DEVELOPMENT	38

3.2.P.2.3.1   INITIAL RISK ASSESSMENTS OF MANUFACTURING PROCESS	38

3.2.P.2.3.2   STUDIES DEVELOPMENT	40

3.2.P.2.3.2.1 Process development	40

3.2.P.2.3.2.2 Updated risk assessment.	43

3.2.P.2.3.2.3 Scale up from Lab to Pilot scale and Commercial scale	44

3.2.P.2.4         CONTAINER CLOSURE SYSTEM	48

3.2.P.2.4.1   PRIMARY PACKAGING MATERIAL	48

3.2.P.2.4.2   SECONDARY PACKAGING MATERIAL	48

3.2.P.2.5         MICROBIOLOGICAL ATTRIBUTES	49

3.2.P.2.6         COMPATIBILITY	50

Annex 1: DSC thermograms of the mixture between excipient and drug substances.	51

Annex 2: Certificates of Analysis	51

LIST OF ABBREVIATIONS:

| API | Active Pharmaceutical Ingredient |
| --- | --- |
| ATC | Anatomical Therapeutic Chemical |
| AV | Acceptance Value |
| BCS | Biopharmaceutics Classification System |
| COA | Certificate Of Analysis |
| CQA | Critical Quality Attribute |
| CU | Content Uniformity |
| CZ | Czech Republic |
| DMF | Drug Master File |
| DS | Drug substance |
| DSC | Differential scanning calorimetry |
| EMA | European Medicines Agency |
| FDA | Food and Drug Administration |
| HPLC | High performance liquid chromatography |
| ICH | International Council for Harmonisation of Technical requirements for Pharmaceuticals for Human Use |
| INN | International Non-proprietary Name |
| IPC | In-process control |
| IR | Intermediate release |
| IUPAC | International Union of Pure and Applied Chemistry Nomenclature |
| MCC | Microcrystalline cellulose |
| NLT | Not less than |
| NMT | Not more than |
| Ph. Eur. | European Pharmacopoeia |
| PSD | Particle size distribution |
| QTPP | Quality Target Product Profile |
| RMP | Reference Medicinal Product |
| RSD | Relative standard deviation |
| SmPC | Summary of Product Characteristics |
| USP | United States Pharmacopoeia |
| UV | Ultraviolet |
| WHO | World Health Organization |
| XRD | X-Ray Diffraction |

# 3.2.P.2.1   	COMPONENTS OF THE DRUG PRODUCT

## 3.2.P.2.1.1 	DRUG SUBSTANCE

### 3.2.P.2.1.1.1	Physicochemical properties

General properties

International Non-proprietary Name (INN)	: <<Information of product>>

IUPAC Name	: <<Information of product>>

CAS registry Number	: <<Information of product>>

Structural formula	: <<Information of product>>

Molecular formula	: <<Information of product>>

Molecular weight	: <<Information of product>>

Appearance	: <<Information of product>>

Melting point	: <<Information of product>>

pKa	: <<Information of product>>

Log P	: <<Information of product>>

Chemical Stability 	: <<Information of product>>

- Thermal degradation	:  <<Information of product>>

- Humidity degradation	:  <<Information of product>>

- Peroxide degradation	: <<Information of product>>

- Acid/ Base degradation	: <<Information of product>>

- Photo degradation	: <<Information of product>>

Particle size distribution:

Table       : Particle size distribution of  <<API name>>.

| Batch No Particle size distribution (µm) | <<Infor from DB>> | <<Infor from DB>> | <<Infor from DB>> |
| --- | --- | --- | --- |
| d(10) | <<Infor from DB>> | <<Infor from DB>> | <<Infor from DB>> |
| d(50) | <<Infor from DB>> | <<Infor from DB>> | <<Infor from DB>> |
| d(90) | <<Infor from DB>> | <<Infor from DB>> | <<Infor from DB>> |

Discussion: << Trình bày tiêu chuẩn cỡ hạt được chọn. Thuyết minh ngắn gọn trích dẫn dữ liệu tại mục 3.2.P.2.2.1.3. Formulation Development Study>>

Flow property:

Table   : Flow properties of  <<API name>>.

| Batch No. Parameters | <<Infor from DB>> | <<Infor from DB>> | <<Infor from DB>> |
| --- | --- | --- | --- |
| Bulk density (g/ ml) | <<Infor from DB>> | <<Infor from DB>> | <<Infor from DB>> |
| Tapped density (g/ ml) | <<Infor from DB>> | <<Infor from DB>> | <<Infor from DB>> |
| Haussler ratio | <<Infor from DB>> | <<Infor from DB>> | <<Infor from DB>> |
| Flow property | <<Infor from DB>> | <<Infor from DB>> | <<Infor from DB>> |

Discussion:

Polymorphism:

Discussion:

<<1. Viết đoạn văn trả lời các câu hỏi sau: Có tồn tại dạng đa hình trong dược phẩm  cụ thể là trong hoạt chất này?/ Dạng thù hình khác nhau có khác biệt trong độ tan, độ ổn định, sinh khả dụng?/ Yếu tố gây chuyển dạng trong sản xuất và độ ổn định?/Phương pháp phân tích để phát hiện các dạng thù hình? 2. Xác định dạng thù hình được chọn, cách kiểm soát nguyên liệu đầu vào và nguy cơ chuyển dạng trong sản xuất và độ ổn định. 3. Kèm sắc ký đồ X-ray/ DSC từ DMF nếu có. **Cần research google, credbile reference (có ghi lại nguồn trích dẫn theo đúng quy định) **.>>

Solubility

Table    : Solubility of << API name>> in different aqueous buffer solutions <<as reported by Jung-Cook et al, nguồn information>>

| Media | Solubility (mg/ ml) |
| --- | --- |
| pH 1.2 | 0.38 ± 0.017 |
| pH 4.5 | 0.31 ± 0.005 |
| pH 6.8 | 0.11 ± 0.002 |

Discussion:

<<Trả lời các câu hỏi sau: Độ tan theo pH sinh lý. Độ ổn định trong pH (nếu có). API tan tốt hay không, có phụ thuộc pH hay không? >>

Permeability and Biopharmaceutical Classification:
<<Các nguồn tài liệu tham khảo liên quan>>

Other important properties:

<< Tính chất hút ẩm Hygroscopicity>>

### 3.2.P.2.1.1.2	Biological properties.

Clinical pharmacology:

3.2.P.2.1.2 	EXCIPIENTS

### 3.2.P.2.1.2.1.    Physicochemical properties

The selection of excipients during the formulation development process was based on the manufacturing techniques to be used, desired product characteristics, components in the reference product and previous experience with similar pharmaceutical forms. The dosages form is based on the reference product (see 3.2.P.2.2.1). All selected excipients are well known and widely used in the pharmaceutical industry. Each excipient is described in the European Pharmacopoeia and these excipients are going to use in the final formulation:

Table      : Description of the excipients in the final formulation.

| No. | Name of excipients | Chemico-physical characters | Excipient grade selection | Reason of selection |
| --- | --- | --- | --- | --- |
| 1. |  |  |  |  |
| 2. |  |  |  |  |
| 3. |  |  |  |  |
| 4. |  |  |  |  |

Table        : Excipients and their recommended range per Handbook of Pharmaceutical Excipients and FDA’s Inactive Ingredients Database

| Name of excipients | Application and recommended range (%) | Function in final formulation | The range in final formulation |
| --- | --- | --- | --- |

### 3.2.P.2.1.2.2.    Excipient compatibility

The chosen excipients for compatibility studies are according to the final formulation of the product. Compatibility study was assessed through DSC analysis of binary mixture of excipient and drug substances at a 1:1 ratio in the solid state. In case ambiguous DSC results were obtained, further analysis using validated HPLC method was carried out on binary mixtures stored at 40 °C/ 75%RH condition for 1 month. The following table shows the scheme of the compatibility study.

Table   : Sample labelling in DSC analysis

| Sample No. | Composition | Ratio |
| --- | --- | --- |

Before running, the DSC equipment was calibrated by reference to Indium and Zinc. The analytical conditions for the drug substance, excipients and mixtures are shown in Table     :

Table      :  Experimental conditions performed on DSC Mettler Toledo model 823 e system and STARe System software.

| Initial temperature | 30.0 oC |
| --- | --- |
| Final temperature | 150.0 oC |
| Temperature increase/ time | 10.0 oC/ min |
| Purge gas | N2 (50.0 mL/ min) |
| Capsule type | Aluminium crucibles with cover |
| Sample weight | 2 – 8 mg |

# Results: DSC thermograms of the mixture are presented in Annex 1.

Comment:

Conclusion:

As the obtained results in this compatibility study, it can be concluded that all excipients are compatible with  API.

# 3.2.P.2.2           DRUG PRODUCT

## 3.2.P.2.2.1        FORMULATION DEVELOPMENT

### 3.2.P.2.2.1.1. Characterization of Reference Product

The product under consideration is in the form of immediate release tablets intended to oral administration which contains the active substances <<API name>>.  API 5 mg film coated tablets is a proposed generic version of the reference drug product Concor (film coated tablets). The objective of the formulation development was to obtain stable reproducible formulations with dissolution profiles that matched with the reference product. Accordingly, the characteristics of the reference drug products are described in Table .

Table : The qualitative and quantitative composition of the reference product in CZ.

| Components | Concor® 5 mg | Concor® 10 mg |
| --- | --- | --- |
| Pharmaceutical form | <<film coated tablets>> | <<film coated tablets>> |

Table 1 : The appearance and physicochemical characteristics of Concor®.

| Characteristics | Concor® 5 mg (batch …….) |  | Concor® 10 mg (batch …….) |  |
| --- | --- | --- | --- | --- |
| Appearance |  |  |  |  |
| Average weight |  |  |  |  |
| Dimensions |  |  |  |  |
| Hardness |  |  |  |  |
| Disintegration time |  |  |  |  |
| Assay |  |  |  |  |
| Impurities |  |  |  |  |
| Content of Uniformity |  |  |  |  |
| Content of Preservatives |  |  |  |  |
| Dissolution profile: - Equipment: - Dissolution media - Agitation speed - Volume of dissolution: | Time (min) | Average dissolution | Time (min) | Average dissolution |

Figure : Dissolution profiles of Concor® 5 mg at pH = 1.2.

### 3.2.P.2.2.1.2. Quality Target Product Profile, CQAs for the Generic Product.

The quality target product profile (QTPP) for ………….. film-coated tablets is given in below. The QTPP summarizes all drug product quality attributes that are needed to ensure quality, safety and efficacy of the drug product for the patient.

#### 3.2.P.2.2.1.2.1 Quality Target Product Profile for the Generic Product:

Based on the clinical and pharmacokinetic (PK) characteristics, in vitro dissolution and physicochemical characteristics of the reference drug product, a quality target product profile (QTPP) was aimed for  API 5/ 10 mg film coated tablets. It is a proposed generic version follow to Concor® 5 mg and Concor® 10 mg marketed by Merck in CZ.

Table 2 : Quality Target Product Profile (QTPP) for generic  API 5/ 10 mg film coated tablets.

| QTPP Elements |  | Target | Justification |
| --- | --- | --- | --- |
| Dosage form |  | Tablet | Pharmaceutical equivalence requirement: same dosage form. |
| Dosage design |  | Immediate release tablet | Immediate release design needed to meet label claims. |
| Dosage strength |  | 10 mg API fumarate 5 mg API fumarate | Pharmaceutical equivalence requirement: same route of administration. |
| Route of administration |  | Oral | Pharmaceutical equivalence requirement: same strength. |
| Pharmacokinetics |  | Bioequivalence (BE) to RMP | Bioequivalence requirement: needed to ensure rapid onset and efficacy. |
| Stability |  | ICH Q1A; At least 24-month shelf-life in the original package. | Equivalent to or better than RMP shelf-life. |
| Drug product quality attributes | Physical Attributes | Pharmaceutical equivalence requirement: Must meet the same compendial or other applicable (quality) standards (i.e., identity, assay, purity, and quality). |  |
|  | Identification |  |  |
|  | Assay |  |  |
|  | Content Uniformity |  |  |
|  | Dissolution |  |  |
|  | Degradation Products |  |  |
|  | Water content |  |  |
|  | Hardness |  |  |
|  | Friability |  |  |
|  | Microbial Limits |  |  |
| Container closure system |  | Container closure system qualified as suitable for this drug product | Needed to achieve the target shelf-life and to ensure tablet integrity during shipping. |
| Administration/ Concurrence with labeling |  | Similar food effect as RMP | RMP labeling indicates that the extent of absorption is not affected by food intake. |
| Alternative methods of administration |  | None | None are listed in the RMP label |

Using the quality target product profile (QTPP) as the starting point, the critical quality attributes (CQAS) of the drug product were identified, and used to guide formulation and process development. CQAS of the film-coated tablets are those that affect the quality targets of physical attributes, drug release, strength, purity, and stability, and thereby drug product efficacy and safety. They should be within an appropriate specification range as defined in the control strategy document in section P.5. The quality attributes of the drug product which were considered critical to the drug product safety and efficacy are listed in Table 3

Table 3: Presentation and justification for CQAs of  API 5/ 10 mg film coated tablets.

| Quality Attributes of the Drug Product |  | Target | Is this a CQA? | Justification |
| --- | --- | --- | --- | --- |
| Physical Attributes | Appearance | Color and shape acceptable to the patient. No visual tablet defects observed. | No | Color, shape and appearance are not directly linked to safety and efficacy. Therefore, they are not critical. The target is set to ensure patient acceptability. |
|  | Odor | No unpleasant odor | No | Neither the drug substance nor the excipients have an unpleasant odor. |
|  | Size | Similar to RMP | No | For comparable ease of swallowing as well as patient acceptance and compliance with treatment regimens, the target for tablet dimensions is set similar to the RMP. |
|  | Score configuration | Unscored for low strength and scored for high strength | Yes* | Score configuration of high strength is to be divided by half. The even breakability of tablet is dependent on the design of punches and hardness of tablets. These CQAs will not be discussed in pharmaceutical development. |
| Identification |  | Positive for API fumarate | Yes* | Though identification is critical for safety and efficacy, this CQA can be effectively controlled by the quality management system and will be monitored at drug product release. Formulation and process variables do not impact identity. Therefore, this CQA will not be discussed during formulation and process development. |
| Assay |  | 100% w/w of label claim | Yes | Assay variability will affect safety and efficacy. Process variables may affect the assay of the drug product. Thus, assay will be evaluated throughout product and process development. |
| Uniformity of dosage units |  | L1 = 15.0, L2 = 25.0 1/ n= 10, AV ≤ L1 or 2/ n= 30, AV ≤ L1 and (1-L2x0.01)M ≤ individual content of the dosage unit ≤ (1+L2x0.01)M | Yes | Variability in content uniformity will affect safety and efficacy. Both formulation and process variables impact content uniformity, so this CQA will be evaluated throughout product and process development. |
| Dissolution |  | ≥ 75% (Q) in 30 minutes in 500 mL of HCl 0.01N using paddle apparatus according to USP at 75 rpm | Yes | Failure to meet the dissolution specification can impact bioavailability. Both formulation and process variables affect the dissolution profile. This CQA will be investigated throughout formulation and process development. |
| Degradation Products |  | Impurity D ≤ 0.5% Maximum individual unknown impurity ≤ 0.2% Total impurities ≤ 1.0% | Yes | Degradation products can impact safety and must be controlled based on compendia/ICH requirements. The target for any unknown impurity is set according to the ICH identification threshold for this drug product. Formulation and process variables can impact degradation products. Therefore, degradation products will be assessed during product and process development. |
| Water content |  | 2.5% - 5.0% | No | Generally, water content may affect degradation and microbial growth of the drug product and can be a potential CQA. However, in this case, water content of input materials and environmental humidity is effectively controlled by the quality management system, hence, the moisture will not impact stability. |
| Hardness |  | ≥ 35 N | No | Generally, hardness and friability affect the intactness of dosage form upon dispensed to the patients. The target is set based upon stability observations to ensure safety and efficacy of the product. This CQA will not be discussed further in pharmaceutical development. |
| Friability |  | ≤ 1.0% |  |  |
| Microbial Limits |  | Meets relevant pharmacopoeia criteria | Yes* | Non-compliance with microbial limits will impact patient safety. However, in this case, the risk of microbial growth is very low because direct compression is utilized for this product. Therefore, this CQA will not be discussed in detail during formulation and process development. |
| * Formulation and process variables are unlikely to impact the CQA. Therefore, the CQA will not be investigated and discussed in detail in subsequent risk assessment and pharmaceutical development. However, the CQA remains a target element of the drug product profile and should be addressed accordingly. |  |  |  |  |

### 3.2.P.2.2.1.3. Formulation Development Study

#### 3.2.P.2.2.1.3.1. Preliminary experiments:

Principle:

Table : Experiment for the selection of excipients.

| Name of ingredient | Grade of selection | API 5 mg film coated tablets (mg) | API 10 mg film coated tablets (mg) |
| --- | --- | --- | --- |

Results:

Table : Results of experiment (………..)

| Parameter |  | Batch ( ) |
| --- | --- | --- |
| Testing result of final blend |  |  |
| Appearance |  |  |
| Moisture content of final blend |  |  |
| Flowability (second/ 100 g) |  |  |
| Testing result of uncoated tablets |  |  |
| Comment in compression process |  | The compression proceeds normal, the tablet weight is relatively uniform. |
| Uniformity of mass (mg) |  | Mean: Max: Min: |
| Uniformity of mass of sub-divided tablets (Breakability) (mg) |  | Mean: Max: Min: |
| Resistance to crushing (N) |  | Mean: Max: Min: |
| Thickness (mm) |  | Mean: Max: Min: |
| Friability (%) |  |  |
| Disintegration time (minutes) |  | Max: Min: |
| Dissolution (Paddles, 900 mL medium pH 1.2, 50 rpm, 37 ± 0.5 oC, % API fumarate dissolved after 30 minutes) |  | Mean: Max: Min: |
| Assay API fumarate (%) |  |  |
| Testing result of film-coated tablets |  |  |
| Uniformity of mass (mg) |  | Mean: Max: Min: |
| Uniformity of mass of sub-divided tablets (Breakability) (mg) |  | Mean: Max Min |
| Resistance to crushing (N) |  | Mean: Max: Min: |
| Thickness (mm) |  | Mean: Max: Min: |
| Disintegration time (minutes) |  | Max: Min: |
| Uniformity of dosage units (content uniformity) |  | Mean: Max: Min: AV = |
| Dissolution (Paddles, 900 mL medium pH 1.2, 50 rpm, 37 ± 0.5 oC, % API fumarate dissolved after 30 minutes) |  | Mean: Max: Min: |
| Assay API fumarate (%) |  |  |
| Related substances (%) | Impurity L |  |
|  | Impurity K |  |
|  | Each unspecified impurity |  |
|  | Total impurities |  |

Conclusion:

Table : Comparison of components of  API film-coated tablets and Concor film-coated tablets

| Ingredients | Concor® | API 5 mg | API 10 mg |
| --- | --- | --- | --- |

#### 3.2.P.2.2.1.3.2. Initial Risk Assessment of Formulation Variables:

A risk assessment of the drug substance attributes was performed to evaluate the impact that each attribute could have on the drug product CQAs. The relative risk that each drug substance attributes presents was ranked as high, medium, or low. This relative risk ranking system was used throughout the pharmaceutical development and is summarized in table .

Table : Overview of relative risk assessment system

| Low | Low risk based on prior knowledge, Broadly acceptable risk. No further investigation is needed. |
| --- | --- |
| Medium | Risk is accepted. Further investigation may be needed in order to reduce the risk. |
| High | Risk is unacceptable. Further investigation is needed to reduce the risk. Controlled strategy applied to high risk to mitigate the risk. |

Table : Initial risk assessment of formulation variables.

| Drug Product CQAs | Formulation Variables |  |  |  |
| --- | --- | --- | --- | --- |
|  | PSD of API | MCC/Lactose Ratio | Calcium hydrogen phosphate (CHP) | Magnesium stearate level |
| Assay | Medium | Medium | Low | Medium |
| Content Uniformity | Medium | Medium | Low | Medium |
| Dissolution | Low | Low | Low | Medium |
| Degradation Products | Low | Low | Low | Low |

Table : Justification for the initial risk assessment of the formulation variables.

| Formulation Variables | Drug Products CQAs | Justification |
| --- | --- | --- |
| PSD of API | Assay |  |
|  | Content Uniformity |  |
|  | Dissolution |  |
|  | Degradation Products |  |
| Microcrystalline cellulose (MCC) level | Assay |  |
|  | Content Uniformity |  |
|  | Dissolution |  |
|  | Degradation Products |  |
| …… | Assay |  |
|  | Content Uniformity |  |
|  | Dissolution |  |
|  | Degradation Products |  |

#### 3.2.P.2.2.1.3.3 Formulation Development Study:

Based on the initial risk assessment of formulation variables, the formulation development studies were conducted:

- Study#1: Examine of API particle size

- Study#2: Examine ratio of disintegrate excipients

- Study#3: …

In these studies, some factors are fixed stably which presented in the following table.

Table : Fixed parameters in study:

| Parameters | Factors |
| --- | --- |
| Production process | Direction compression |
| Summary process | Homogeneous mixture of a, b,c Add d, Final blending Compression, Coating |
| Components | Calcium hydrogen phosphate (Dicafos A60), Microcrystalline cellulose (MCC 102),… |
| Batch No | 1.000 tablets |
| Weight | 200 mg |
| Appearance | White, round tablet 8 mm, engraved on one side, flat-faces |

Study #1:

Table : Composition of trial batch

| Batch No. | 01/001 |  | 01/002 |  | 01/003 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Batch size | (mg) | % | (mg) | % | (mg) | % |
| Core tablet |  |  |  |  |  |  |
| API fumarate |  |  |  |  |  |  |
| Calcium hydrogen phosphate (Dicafos A60) |  |  |  |  |  |  |
| Total 1 |  |  |  |  |  |  |

Results:

Table : Result of trial batch 01/001, 01/002, 01/003.

| Batch no. Parameter |  | 01/001 |  | 01/002 | 01/003 |
| --- | --- | --- | --- | --- | --- |
| Testing result of final blend |  |  |  |  |  |
| Appearance |  |  |  |  |  |
| Moisture content of final blend |  |  |  |  |  |
| Flow ability (second/ 100 g) |  |  |  |  |  |
| Testing result of uncoated tablets |  |  |  |  |  |
| Comment in compression process |  | The compression proceeds normal, the tablet weight is relatively uniform. |  |  |  |
| Uniformity of mass (mg) |  | Mean: Max: Min: |  |  |  |
| Uniformity of mass of sub-divided tablets (Breakability) (mg) |  | Mean: Max: Min: |  |  |  |
| Resistance to crushing (N) |  | Mean: Max: Min: |  |  |  |
| Thickness (mm) |  | Mean: Max: Min: |  |  |  |
| Friability (%) |  |  |  |  |  |
| Disintegration time (minutes) |  | Max: Min: |  |  |  |
| Dissolution (Paddles, 900 mL medium pH 1.2, 50 rpm, 37 ± 0.5 oC, % API fumarate dissolved after 30 minutes) |  | Mean: Max: Min: |  |  |  |
| Assay API fumarate (%) |  |  |  |  |  |
| Testing result of film-coated tablets |  |  |  |  |  |
| Uniformity of mass (mg) |  | Mean: Max: Min: |  |  |  |
| Uniformity of mass of sub-divided tablets (Breakability) (mg) |  | Mean: Max Min |  |  |  |
| Resistance to crushing (N) |  | Mean: Max: Min: |  |  |  |
| Thickness (mm) |  | Mean: Max: Min: |  |  |  |
| Disintegration time (minutes) |  | Max: Min: |  |  |  |
| Uniformity of dosage units (content uniformity) |  | Mean: Max: Min: AV = |  |  |  |
| Dissolution (Paddles, 900 mL medium pH 1.2, 50 rpm, 37 ± 0.5 oC, % API fumarate dissolved after 30 minutes) |  | Mean: Max: Min: |  |  |  |
| Assay API fumarate (%) |  |  |  |  |  |
| Related substances (%) | Impurity L |  |  |  |  |
|  | Impurity K |  |  |  |  |
|  | Each unspecified impurity |  |  |  |  |
|  | Total impurities |  |  |  |  |

Conclusion:

Study #2

Study #3

#### 3.2.P.2.2.1.3.4. Updated risk assessment of the formulation variables.

Based on the results of the formulation development studies, the risk assessment of the formulation variables was updated as given in Table with justifications provided in Table   .

Table : Updated risk assessment of the formulation variables.

| Drug Product CQAs | Formulation Variables |  |  |  |
| --- | --- | --- | --- | --- |
|  | PSD of API | MCC/Lactose Ratio | Calcium hydrogen phosphate (CHP) | Magnesium stearate level |
| Assay | Low | Low | Low | Low |
| Content Uniformity | Low | Low | Low | Low |
| Dissolution | Low | Low | Low | Low |
| Degradation Products | Low | Low | Low | Low |

Table : Justification for the reduced risks of the formulation variables

| Formulation Variables | Drug Products CQAs | Justification |
| --- | --- | --- |
| Microcrystalline cellulose (MCC) level | Assay |  |
|  | Content Uniformity |  |
| … | Assay |  |
|  | Content Uniformity |  |
|  | Dissolution |  |
|  | Degradation Products |  |

#### 3.2.P.2.2.1.3.5. Final formulation:

The formula is finalized as below:

| No. | Name of ingredient | Function | API 5 mg film coated tablets |  | API 10 mg film coated tablets |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | mg | % | mg | % |
|  | API fumarate | Active ingredient |  |  |  |  |
|  | Calcium hydrogen phosphate | Filler |  |  |  |  |
|  | Magnesium stearate | Lubricant |  |  |  |  |
| Total 1 |  | - |  |  |  |  |
|  | Hypromellose | Film-forming agent |  |  |  |  |
|  | Purified water | Solvent | q.s | - | q.s | - |
| Total 2 |  | - |  |  |  | - |

Discussion: The final formulation is the same as the commercial batch formulation that are described in previous section 3.2.P.1 and also similar as biobatch formulation.

3.2.P.2.2.2. OVERAGES

Not applicable. Overages are not used in the formulation of Name product.

3.2.P.2.2.3 PHYSICOCHEMICAL AND BIOLOGICAL PROPERTIES

### 3.2.P.2.2.3.1. Development of a dissolution method

#### 3.2.P.2.2.3.1.1 Sink condition.

In principle, pre-requisite for the development of dissolution test is that the medium selected must satisfy sink condition. According to Ph. Eur., sink condition is defined as in such a manner that the material already in solution does not exert a significant modifying effect on the rate of dissolution of the remainder. Sink conditions normally occur in a volume of dissolution medium that is at least 3 - 10 times the saturation volume. Solubility of  API over physiological pH range studied by Jung-Cook et al was summarized in table below.

Table : Sink conditions study of  API in different solution medium.

| Medium | Minimum theoretical concentration to meet sink conditions (mg/mL) | Experimental concentration (mg/mL) |
| --- | --- | --- |
| pH 1.2 | 0.03 | 0.38 ± 0.017 |
| pH 4.5 | 0.03 | 0.31 ± 0.005 |
| pH 6.8 | 0.03 | 0.11 ± 0.002 |

Conclusion: The experimental concentration are over the minimum theoretical concentration. Sink condition are met for  API at the following media pH 1.2, 4.5, 6.8.

#### 3.2.P.2.2.3.1.2 Dissolution method

In vitro dissolution test is an indispensable part for routine quality control of product by measuring the release active substance from its dosage form. The aspects of the method are carefully evaluated to make appropriate choices. The chosen dissolution method was validated according to the CPMP/ICH-Note for Guidance on Validation of Analytical Procedures and mentioned in section 3.2.P.5. The table below summaries dissolution study from pharmacopoeia, experiment and selected method. The selected method bases on the result of experiment, which mention in result section.

Table : Summary of dissolution method:

|  | USP | BP | Experiment | Selected method |
| --- | --- | --- | --- | --- |
| Apparatus | II |  | II | II |
| Stirring speed (rpm) | 75 |  | 50 - 100 | 75 |
| Medium | Hydrochloric acid 0.01N |  | Hydrochloric acid 0.01N | Hydrochloric acid 0.01N |
| Volume | 900 ml |  | 500 – 900 ml | 500 ml |

Results:

Table : Summarized results of experimental batches 01/001 at different stirring speeds are shown below:

| Specification |  | Batch no. 01/001 |  |  |
| --- | --- | --- | --- | --- |
| Stirring speed (rpm) |  | 50 | 75 | 100 |
| Dissolution in 500 ml HCl 0.01N (% dissolved) | 0 min | 0 |  |  |
|  | 5 min | 80.28 |  |  |
|  | 15 min | 91.01 |  |  |
|  | 30 min | 95.30 |  |  |
|  | 45 min | 96.79 |  |  |
|  | 60 min | 97.91 |  |  |

Figure . Comparison of dissolution profiles of 01/001 at different stirring speeds.

Comments:

#### 3.2.P.2.2.3.1.2 Discriminatory power of the method.

Method:

Following the Reflection paper on the dissolution specification for generic solid oral immediate release products with systemic action (EMA/CHMP/CVMP/QWP/336031/2017), “batches with meaningful changes compared to the applied finished product should be manufactured. Such changes may relate to the quantitative formulation, material specifications and/or using slightly modified process parameters”.

In order to evaluate the discriminatory power of the dissolution method, attempts were made to modify (the ratio of disintegrate excipients, hardness, particle size of API) and to see whether the dissolution method is capable of detecting such variations.

Table : Description of study:

|  | Changing objects | Test information |  |  |
| --- | --- | --- | --- | --- |
| Experiment 1 | The ratio of excipients to disintegrate (%) | 0.5 (01/001) – Current formular | 1 (01/002) | 2 (01/003) |
| Experiment 2 | Hardness (N) | 50 (01/001) – Current formular | 100 (01/001) – f1 | 150 (01/001) – f2 |
| Experiment 3 | Particle size of API, d90 (µm) | 50 | 100 | 200 |

Results:

Table : Summarized results of experimental batches.

|  |  | The ratio of excipients to disintegrate (%) |  |  | Hardness (N) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Batch No Specification |  | (01/001) – Current formular | 01/002 | 01/003 | (01/001) – Current formular | 01/001 –f1 | 01/001 – f2 |
| Disintegration time Max Min |  |  |  |  |  |  |  |
| Dissolution in 500 ml HCl 0.01N (% amlodipine dissolved) | 0 min |  |  |  |  |  |  |
|  | 5 min |  |  |  |  |  |  |
|  | 15 min |  |  |  |  |  |  |
|  | 30 min |  |  |  |  |  |  |
|  | 45 min |  |  |  |  |  |  |
|  | 60 min |  |  |  |  |  |  |

Figure : The dissolution profiles for different ratio of excipients to disintegrate (%)

Comment: As it is shown on the graphic above, dissolution is observed between the 3 batches with different the ratio of Name excipients.

Conclusion:

The selected dissolution method is capable of distinguishing.

##### 3.2.P.2.2.3.1.1.3 Comparative dissolution profile

Comparative dissolution between different scale batches of  API 10 mg: (Table….)

Table : Comparative dissolution profile at the main dissolution medium.

| Average dissolution (%) ± SD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Time (min.) | Apparatus: paddles Medium: 500 mL medium HCl 0.01 N |  |  |  | Rotation speed: 50 rpm Temperature: 37 ± 0.5 oC |  |
|  | Lab batch | Pilot batch | Validation batch 1 | Validation batch 2 |  | Validation batch 3 |
| 5 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |
| f2 |  |  |  |  |  |  |

Figure : Dissolution profile of  API 10 mg film coated tablets vs Concor® 10 mg batch no. G01APM in medium HCl 0.1 N.

Conclusion:

From the results of comparative dissolution profiles, biobatch and  API 10 mg film coated tablets scales are equivalent dissolution rate that more than 85% in 15 mins/ f2 > 50.

Comparative dissolution between biobatches in main medium: (Table )

Table : The dissolution profile at HCl 0.01 N.

| Average dissolution (%) ± SD |  |  |
| --- | --- | --- |
| Time (min.) | Apparatus: paddles Medium: 500 mL medium HCl 0.01 N | Rotation speed: 50 rpm Temperature: 37 ± 0.5 oC |
|  | RMP (Biobatch) | API 10 mg film coated tablets (Biobatch) |
| 5 | 79.75 ± 11.65 |  |
| 10 | 86.50 ± 6.71 |  |
| 15 | 94.73 ± 2.71 |  |
| f2 |  |  |

Figure : The dissolution profile of  API 10 mg film coated tablets trial batch no. 03/001 vs Concor® 10 mg batch no. G01APM in 500 mL medium HCl 0.01 N.

Conclusion:

The results of the study of the comparative dissolution profiles between Concor® and  API 5 mg film coated tablets show that both formulations have a similar behavior at all medium.

Dissolution specification:

The result of API 10 mg, batch 01/001 (biobatch) is 90% after 30 minutes. Moreover, the selected standard: NLT 85 % (Q=80 %) of the labelled amount of API dissolved in 30 minutes.

Comparative dissolution between biowaiver (dose 2.5, 5, 10 mg): (Table )

Table : The dissolution profile at HCl 0.01 N

| Average dissolution (%) ± SD |  |  |
| --- | --- | --- |
| Time (min.) | Apparatus: paddles Medium: 500 mL medium HCl 0.01 N | Rotation speed: 50 rpm Temperature: 37 ± 0.5 oC |
|  | API 10 mg, batch 01/001 (biobatch) | API 5 mg, batch 01/001 (commercial batch) |
| 5 | 79.75 ± 11.65 |  |
| 10 | 86.50 ± 6.71 |  |
| 15 | 94.73 ± 2.71 |  |
| f2 |  |  |

Figure: The dissolution profile of  API 10 mg film coated tablets trial batch no. 03/001 and Concor® 10 mg batch no. G01APM in 500 mL medium HCl 0.01 N.

Conclusion:

The results of the study of the comparative dissolution profiles between  API 5 mg and  API 10 mg show that both strengths have a similar behavior at all medium.

### 3.2.P.2.2.3.2. Impurities profile of tablets

Comparative impurity profile for Concor® and  API 10 mg film coated tablets.

Object

Verify that the impurities of API tablets developed by STELLAPHARM-VN are consistent with those quantitated in the drug product of the reference drug product Concor®. The impurity profile was obtained using batches of API tablets/Concor®, used in the bioequivalence study (see Module 5, for more information).

Procedure

Samples:

| Name of product | Batch | Mfg date / Expiry date | Analysis |
| --- | --- | --- | --- |
| Concor® | Batch 24753 (biobatch) Batch 01/001 Batch …. |  |  |
| API 10 mg film coated tablets | Batch X003 (biobatch) |  |  |

Methods:

The analytical method appears in section 3.2.P.5.2

Results:

Chromatograms

Figure 1, Figure 2,…

Quantitative results:

Peak area % corresponding to chromatograms of Concor® and API 10 mg film coated tablets are summarised in the following table:

| Impurity | Concor® | API 10 mg film coated tablets |
| --- | --- | --- |

Discussion and Conclusion:

# 3.2.P.2.3 MANUFACTURING PROCESS DEVELOPMENT

As discussed in Section 3.2.P.2.2.1.3.3 Process Selection, direct compression was chosen as the suitable manufacturing method for  API 5/ 10 mg film-coated tablets. Figure presents the process map for the finalized formulation of Generic , 20 mg.

Figure : Process flow diagram describes the expected manufacturing process divided into several steps.

## 3.2.P.2.3.1. INITIAL RISK ASSESSMENTS OF MANUFACTURING PROCESS

Based on the previous experience in section 3.2.P.2.2.1.3 Formulation Development Study, the degree of risk associated with entire manufacturing process of  API 5 & 10 mg tablets was determined. The initial risk assessment of the overall manufacturing process is shown in Table ….. and justifications are provided in Table….. Previous experience with these process steps was used to determine the degree of risk associated with each process step and its potential to impact the CQAs of the finished drug product.

Table : Initial risk assessment of the manufacturing process of  API 5 & 10 mg tablets

| Drug Product CQAs | Process Steps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Blending 1 | Blending 2 | Blending 3 | Final blending | Compression | Film-coating |
| Assay | Low | Low | Low | Medium | Medium | Low |
| Content Uniformity | Low | Low | Low | Medium | Medium | Low |
| Dissolution | Low | Low | Low | Medium | Medium | Low |
| Degradation Products | Medium | Medium | Medium | Medium | Medium | Medium |

Table : Justification for the initial risk assessment of the manufacturing process

| Process Steps | Drug Products CQAs | Justification |
| --- | --- | --- |
| Blending | Assay | In this step, sieved materials are added and blended. It will make next steps more effective and bring low risk to product CQAs. The risk is low |
|  | Content Uniformity |  |
|  | Dissolution |  |
|  | Degradation Products | It was observed that API is not stable when exposure to high humid environmental condition in a long time period. The risk is medium. |
| Final blending | Assay | In this step, sieved materials are added and blended. A not well homogenized mixture can cause troubles in assay and CU. The risk is medium. |
|  | Content Uniformity |  |
|  | Dissolution | It is known that over mixed blends into which magnesium stearate is added could slow down the dissolution. The risk is medium. |
|  | Degradation Products | It was observed that API is not stable when exposure to high humid condition in a long time period. The risk is medium. |
| Compression | Assay | Excessive tablet weight variability can result in unacceptable assay and CU. The risk is medium. |
|  | Content Uniformity |  |
|  | Dissolution | Tablet hardness and disintegration time can significantly impact dissolution. The API is highly soluble, thus the risk is medium. |
|  | Degradation Products | It was observed that API is not stable when exposure to high humid condition in a long time period. The risk is medium. |
| Film-coating | Assay | Film coating is only for cosmetic purposes and in such small amount that it is unlikely to have an impact on assay, CU, dissolution. The risk is low. |
|  | Content Uniformity |  |
|  | Dissolution |  |
|  | Degradation Products | It was observed that API is not stable when exposure to high humid condition in a long time period. The risk is medium. |

## 3.2.P.2.3.2. STUDIES DEVELOPMENT

### 3.2.P.2.3.2.1 Process development

Based on the initial risk assessment of the manufacturing process, the process development studies were conducted:

- Study#4: Examine final blending step

- Study#5: Examine wet mixing step

- Study#6: …

In these studies, some factors are stably fixed which presented in the following table.

Table : Fixing parameters in study.

| Parameters | Factors |
| --- | --- |
| Production process | Direction compression |
| Summary process | Homogeneous mixture of a, b,c Add d, Mix complete Compression, Coating |
| Final formulation | The finalized formula see in table … |
| … |  |

Study #4:

Table : Experimental design on final blending time of  API 5 mg film coated tablets

| API 5 mg film coated tablets |  |  |  |  |
| --- | --- | --- | --- | --- |
| Batch no. |  | Batch 0010321 | Batch 0020321 | Batch 0030321 |
| Batch size |  | 34.0 kg # 200,000 tablets | 34.0 kg # 200,000 tablets | 34.0 kg # 200,000 tablets |
| Homogenous blending 1 | Speed (rpm) | 10 | 10 | 10 |
|  | Time (minutes) | 10 | 10 | 10 |
| Final blending | Speed (rpm) | 10 | 10 | 10 |
|  | Time (minutes) | 1, 3, 5 | 1, 3, 5 | 1, 3, 5 |

The taken sampling process: After specified blending intervals, the mixture was sampled for measurement and assessment of uniformity of API content. The other intrinsic characteristics of the blend, including appearance, flowability, bulk density, tapped density, particle size distribution and loss on drying were examined on the withdrawn samples at 5 minutes. Details of the quantity and points of sampling are designated in Figure….:

| Quantity of samples | Amount of each sample (g) | Test |
| --- | --- | --- |
| 1 | 100 | Flowability, bulk density, tapped density, particle size distribution, loss on drying. |
| 10 | 10 | Appearance, content uniformity. |

Results:

Table : Results on blending process development of  API 5 mg film coated tablets).

|  | Test |  | Specification |  | Results |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Batch 0010321 | Batch 0020321 | Batch 0030321 |
| 1. | Flowability (sec/ 100 grams) |  | Defining the acceptable range |  | 8.46 | 8.35 | 8.46 |
| 2. | Bulk density (g/ ml) |  | Defining the acceptable range |  | 0.73 | 0.74 | 0.74 |
| 3. | Tapped density (g/ ml) |  | Defining the acceptable range |  | 0.86 | 0.85 | 0.86 |
| 4. | Haussler ratio |  | / |  | 1.19 | 1.15 | 1.16 |
| 5. | Particle size distribution |  | / | ≥ 425 m | 2.00 | 3.04 | 3.05 |
|  |  |  |  | 250 – 425 m | 3.70 | 3.52 | 4.45 |
|  |  |  |  | 150 – 250 m | 30.96 | 29.88 | 30.07 |
|  |  |  |  | 75 – 150 µm | 22.76 | 24.01 | 19.89 |
|  |  |  |  | < 75 µm | 40.30 | 39.55 | 42.54 |
| 6. | Loss on drying (%) |  | ≤ 2.0% w/w at 100 oC |  | 1.42 | 1.54 | 1.53 |
| 7. | Content Uniformity (% API fumarate in 100 mg final blend) | 1st min | Average: 2.94% ± 5% (2.79% - 3.09%) |  | Mean: 2.96 Max: 2.99 Min: 2.92 RSD: 0.93 | Mean: 2.98 Max: 3.06 Min: 2.92 RSD: 1.68 | Mean: 2.92 Max: 2.98 Min: 2.85 RSD: 1.70 |
|  |  |  | RSD ≤ 5% |  |  |  |  |
|  |  | 3rd min | Average: 2.94% ± 5% (2.79% - 3.09%) |  | Mean: 2.95 Max: 3.00 Min: 2.92 RSD: 1.08 | Mean: 3.01 Max: 3.04 Min: 2.97 RSD: 1.04 | Mean: 3.00 Max: 3.05 Min: 2.95 RSD: 0.36 |
|  |  |  | RSD ≤ 5% |  |  |  |  |
|  |  | 5th min | Average: 2.94% ± 5% (2.79% - 3.09%) |  | Mean: 2.98 Max: 3.01 Min: 2.94 RSD: 0.94 | Mean: 2.97 Max: 3.00 Min: 2.92 RSD: 0.99 | Mean: 3.01 Max: 3.02 Min: 2.99 RSD: 0.36 |
|  |  |  | RSD ≤ 5% |  |  |  |  |

Conclusion:

### 3.2.P.2.3.2.2 Updated risk assessment.

The risks identified during the initial assessment of the compression step were reduced through development studies. The updated risk assessment is presented in Table    .

Table : Updated risk assessment of .

| Drug Product CQAs | Manufacturing process variables |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mixing 1 | Sieving 1 | Mixing 2 | Mixing 3 | Sieving 2 | Final mixing | Tablet compression |
| Assay | Low | Low | Low | Low | Low | Low | Low |
| Content Uniformity | Low | Low | Low | Low | Low | Low | Low |
| Dissolution | Low | Low | Low | Low | Low | Low | Low |
| Degradation Products | Low | Low | Low | Low | Low | Low | Low |

Table : Justification for the updated risk assessment of overall manufacturing process

| Manufacturing process variables | Drug Products CQAs | Justification |
| --- | --- | --- |
| Mixing 1 | Assay | With the homogenization time was investigated, no problems impact to content uniformity and assay were reported during experiment. This is low risk. |
|  | Content Uniformity |  |
| Final mixing | Assay | With the homogenization time was investigated, no problem impact to assay was reported during experiment. The risk is reduced from medium to low. |
|  | Content Uniformity |  |
|  | Dissolution | No problems found to dissolution after mixing Mgst during experiment. The risk is reduced from medium to low. |
| Tablet compression | Assay | Compression force and press speed are investigated in the project and the resulting did not adversely affect assay and content uniformity when checked the process. The risk is reduced from medium to low. |

### 3.2.P.2.3.2.3 Scale up from Lab to Pilot scale and Commercial scale

Process development was conducted on the lab scale (1.7 – 8.5 kg). This section describes the principles used to scale-up the process to the pilot scale (34.00 kg). The same principles will be employed to scale-up the process to the commercial scale (300.00 kg). The table below summarizes the different process scales.

Table : Scale up from laboratory scale to pilot scale to commercial scale (proposed) of both strength 5 mg & 10 mg.

| Scales Stages |  | Laboratory scale | Pilot scale |  | Proposed range for commercial scale |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Batch No.: 01/001 | Batch No.: 011213 |  | Batch No.: NA |  |
|  |  |  | 011213/AML5 | 011213/AML10 |  |  |
|  |  | 10 mg | 5 mg | 10mg | 5 mg | 10 mg |
|  |  | 5,000 tablets | 250,000 tablets | 125,000 tablets | 2,000,000 tablets | 1,000,000 tablets |
| Mixing 1 | Speed (rpm) | NA | 10 | 10 | 10 | 10 |
|  | Time (minutes) | 3 | 30 | 30 | 30 | 30 |
| Mixing 2 | Speed (rpm) | NA | 10 | 10 | 10 | 10 |
|  | Time (minutes) | 5 | 15, 20 | 15, 20 | 20* | 20* |
| Mixing 3 | Speed (rpm) | NA | 10 | 10 | 10 | 10 |
|  | Time (minutes) | 3 | 20, 25, 30 | 20, 25, 30 | 30* | 30* |
| Final mixing | Speed (rpm) | NA | 10 | 10 | 10 | 10 |
|  | Time (minutes) | 2 | 6 | 6 | 6 | 6 |
|  | Blending machine | Double BE bags | Pillar type bin blender 300 L | Pillar type bin blender 300 L | Pillar type bin blender 600 L Octagonal blender 2200 L | Pillar type bin blender 600 L Octagonal blender 2200 L |
| Compression | Hardness | 65 – 130 N | 65 – 130 N (optimum range: 90 – 100 N) | 65 – 130 N (optimum range: 90 – 100 N) | 90 – 100 N* | 90 – 100 N* |
|  | Speed | 4,800 tablets/ hour | 140,000 – 230,000 tablets/ hour (optimum speed: 200,000 tablets/ hour) | 100,000 – 220,000 tablets/ hour (optimum speed: 200,000 tablets/ hour) | 200,000 tablets/ hour* | 200,000 tablets/ hour* |
|  | Machine | Karnavati tablet press machine 16 stations (4 stations used) | IMA KILIAN tablet press machine 63 stations 700B | IMA KILIAN tablet press machine 63 stations 700B | IMA KILIAN tablet press machine 63 stations 700B | IMA KILIAN tablet press machine 63 stations 700B |

(*) Parameters to be re-validated at commercial scale

Comparison table of the analytical results of tablet in different batch sizes is summarized as below:

| Scale Parameters | Lab | Pilot |  |
| --- | --- | --- | --- |
|  | Batch | Batch | Batch |
| Assay (%) 95.0-105.0% | 100.1 | 100.03 | 100.35 |
| Uniformity of dosage units | AV: 3.41 | AV: 4.37 | AV: 4.22 |
| Dissolution (%) In 30 minutes ≥ 80%(Q) | 100.7 (98.8 – 102.0%) | 99.7 (97.9 – 100.6%) | 100.2 (98.2 – 101.0% |
| Related substances (%) Impurity L ≤ 0.5% Impurity K ≤ 0.5% Each unspecified impurity ≤ 0.2 % Total impurities ≤ 1.5% | 0.03 0.07 0.06 0.26 | 0.04 0.05 0.07 0.21 | 0.03 0.06 0.09 0.27 |

Conclusion:

# 3.2.P.2.4 CONTAINER CLOSURE SYSTEM

## 3.2.P.2.4.1. PRIMARY PACKAGING MATERIAL

| Packaging material | PVC-PE-PVDC/ Aluminium foil blister is equivalent to material of RMP |
| --- | --- |
| Description | PVC/ PE/ PVDC foil, 250 µm/ 30 µm/ 90 g/m2 opaque Aluminium foil 25 µm |
| Suitability | Protection: Compatibility: Conclusion: |
| Quality Control | The specification of primary packaging material is detailed in the section 3.2.P.7. |
| Stability | The stability data of the product is detailed in the section 3.2.P.8 |

## 3.2.P.2.4.2. SECONDARY PACKAGING MATERIAL

Description: Carton box containing the labelling information.

Dose Calculation Ratio and Proportion Method.

# 3.2.P.2.5 MICROBIOLOGICAL ATTRIBUTES

The captioned drug product is non-sterile solid oral dosage form.

According to the ICH Q6A Specifications: Test Procedures and Acceptance Criteria for New Drug Substances and New Drug Products- Chemical Substances, microbial contamination testing is seen as an attribute of Good Manufacturing Practice, as well as of quality assurance. As mentioned in Ph.Eur. general chapter Tablets, in the manufacture, packaging, storage and distribution of tablets, suitable measures are taken to ensure their microbiological quality; recommendations on this aspect are provided in general chapter 5.1.4. Microbiological quality of non-sterile pharmaceutical preparations and substances for pharmaceutical use.

Adopted from these recommendations to this product as well as for all products in non-sterile solid oral dosage form, the general strategy applied in factory for control the microbial contamination is as below.

Necessary measures in accordance with EU GMP principles are performed and controlled in factory during manufacturing process, packaging and storage to ensure general product quality and microbial contamination.

Microbiological test is evaluated in the initial process validation of product and continuously controlled internally as a GMP practice from the first batch of year and periodical testing of each 10 production batches on the year.

Microbial contamination acceptance criteria required for the product are set in accordance with that applied to non-aqueous preparations for oral use as described in the Table 5.1.4.-1 of Ph. Eur. general chapter 5.1.4 as following.

Total aerobic microbial count (TAMC) ≤ 103 CFU/g

Total combined yeasts/moulds count (TYMC) ≤ 102 CFU/g

Escherichia coli: Absent/g

The analytical procedure follows the Ph.Eur. general chapter 2.6.12 & 2.6.13.

The result of the microbial contamination tested during the process validation on commercial batches of this product well complies with the aforementioned specification at the minimum level and summarized as below:

| Batch number |  |  |  |
| --- | --- | --- | --- |
| Batch size |  |  |  |
| Results of microbial contamination: TAMC TYMC Escherichia coli |  |  |  |

# 3.2.P.2.6 COMPATIBILITY

This section is not applicable since the drug product is not reconstituted with a diluent.

# Annex 1: DSC thermograms of the mixture between excipient and drug substances.

# Annex 2: Certificates of Analysis