# Formulation Trial 01 — Bisoprolol 5 mg Tablet (Wet Granulation)

## Trial Overview

| Attribute | Value |
| --- | --- |
| Trial ID | BIS-5-WG-001 |
| Formulation code | F-01 |
| Active ingredient | Bisoprolol fumarate 5 mg |
| Granulation method | Wet granulation |
| Batch size | 10,000 tablets |
| Date | 2025-06-15 |
| Operator | Nguyễn Văn A |

## Formulation Composition

| # | Ingredient | mg/tablet | % w/w | Function |
| --- | --- | --- | --- | --- |
| 1 | Bisoprolol fumarate | 5.00 | 5.00 | Active |
| 2 | Microcrystalline cellulose (PH-102) | 55.00 | 55.00 | Filler/binder |
| 3 | Lactose monohydrate (200 mesh) | 25.00 | 25.00 | Filler |
| 4 | Povidone K30 (binder solution 5% w/v) | 5.00 | 5.00 | Binder |
| 5 | Croscarmellose sodium | 3.00 | 3.00 | Disintegrant |
| 6 | Colloidal silicon dioxide | 1.00 | 1.00 | Glidant |
| 7 | Magnesium stearate | 1.00 | 1.00 | Lubricant |
| Total |  | 95.00 | 100.00 |  |

## Coating

| Attribute | Value |
| --- | --- |
| Coating material | Opadry II White (hypromellose, titanium dioxide, macrogol) |
| Coating weight gain | 3.0% w/w |
| Coating pan | 12-inch, inlet temp 60°C |

## Results

| Test | Specification | Result | Pass/Fail |
| --- | --- | --- | --- |
| Appearance | White, round, biconvex film-coated tablet | Conforms | Pass |
| Average weight (n=20) | 95.0 mg ± 5% | 94.8 mg (RSD 1.2%) | Pass |
| Hardness (n=10) | 40–100 N | 68 N (range 58–78) | Pass |
| Friability (n=20) | ≤ 1.0% | 0.32% | Pass |
| Disintegration | ≤ 15 min | 4 min 20 sec | Pass |
| Assay (HPLC) | 95.0–105.0% | 99.2% | Pass |
| Content uniformity (n=10) | AV ≤ 15.0 | AV = 6.8 | Pass |
| Dissolution (Q=80%, 30 min, n=6) | Each unit ≥ 80% | 92%, 95%, 91%, 94%, 93%, 96% | Pass |
| Dissolution mean | — | 93.5% | — |
| Related substances | Individual ≤ 0.5%; Total ≤ 1.0% | Max individual 0.12%; Total 0.35% | Pass |
| Loss on drying | ≤ 2.0% | 0.8% | Pass |

## Observations

- Wet granulation produced good flow properties (Carr index 18%).

- Dissolution profile shows rapid release (>80% at 15 min).

- Hardness distribution is tight, indicating consistent compression.

- No sticking or picking observed during compression.